Q1. Examine that which of the following is a two-byte instruction?

- MOV

- MVI

- ADD

- CMA

Answer-MVI

Q2. Following instruction is executed in 8085, LDB 4000H.

Choose the correct statement

- 4000H is copied to register B

- 4002H is copied to register B

- Data at 4000H is copied to register B

- Data at 4000H is copied to register B and location 4000H will be cleared

Answer- Data at 4000H is copied to register B

Q3. The content of “A” after executing the following program is:

X= 20, Y= 40

MOV A, X

ADD A, Y

MOV Y, A

- A= 20

- A= 40

- A= 50

- A= 60

Answer- A=60

Q4. The 8085 assembly language instruction that stores the content of H and L registers into memory locations 2050H and 2051H respectively, is

- SPHL 2050H

- SPHL 2051H

- SHLD 2050H

- STAX 2051H

Answer- SHLD 2050H

Q5. In a microprocessor, the address of the next instruction to be next executed, is stored in

- Stack Register

- Sequence Register

- Stack Pointer

- Program Counter

Answer- D

Q6. The main purpose of Accumulator Register in 8085 is

- Storage of Data temporarily

- Permanent storage of data

- Permanent storage of addresses

- Storage of Instructions

Answer- Storage of Data temporarily

Q7. Following Examples are applied in which Addressing mode of CPU:

- MVI B 45 (move the data 45H immediately to register B)

- LXI H 3050 (load the H-L pair with the operand 3050H immediately)

- JMP address (jump to the operand address immediately)

- Implied Mode

- Register Mode

- Immediate Mode

- Indirect Register Mode

Answer-Immediate Mode

Q8. Following Examples are applied in which Addressing mode of CPU:

- MOV A, B (move the contents of register B to register A)

- ADD B (add contents of registers A and B and store the result in register A)

- INR A (increment the contents of register A by one)

- Implied Mode

- Register Mode

- Immediate Mode

- Indirect Register Mode

Answer-Register Mode

Q9. Following Examples are applied in which Addressing mode of CPU:

- LDA 2050 (load the contents of memory location into accumulator A)

- LHLD address (load contents of 16-bit memory location into H-L register pair)

- Implied Mode

- Register Mode

- Immediate Mode

- Direct Addressing Mode

Answer- Direct Addressing Mode

Q10. Following Examples are applied in which Addressing mode of CPU:

- CMA (finds and stores the 1’s complement of the contents of accumulator A in A)

- RRC (rotate accumulator A right by one bit)

- RLC (rotate accumulator A left by one bit)

- Implied/implicit Mode

- Register Mode

- Immediate Mode

- Indirect Register Mode

Answer- Implied/implicit Mode

Q.11 Following Examples of Instruction format have how many addresses ?

LOAD   A            /*  AC <- M[A]   */

ADD     	B            /*  AC <-AC + M[B]  */

STORE T            /*  M[T] <- AC   */

- One Address

- Two Address

- Three Address

- Zero Address

Answer- One address

Q12. Following Examples of Instruction format have how many addresses?

MUL  X, R1, R2                /*     M[X] <- R1 * R2     */

- One Address

- Two Address

- Three Address

- Zero Address

Answer- Three Address