Worksheet of Unit-4

(Micro operations (Arithmetic, Logic, Shift), Addition, Subtraction, Multiplication Algorithm, Booth’s Multiplication)

Q1. Booth’s multiplication algorithm is mainly used to:

- Reduce number of shifts

- Handle signed numbers efficiently

- Eliminate carry

- Increase memory size

Q2. In Booth’s algorithm, when the bit pair (Q₀, Q₋₁) = (1, 0), the operation is:

(a) Add multiplicand
(b) Subtract multiplicand
(c) No operation
(d) Shift left

Q3. Which condition indicates overflow in signed addition?

(a) Carry out = 1
(b) Carry in ≠ Carry out of MSB
(c) Result = 0
(d) MSB = 1

Q4. A register contains 00000000. After decrement operation, the result becomes:

(a) 00000001
(b) 11111111
(c) 00000000
(d) 10000000

Q5. Apply arithmetic right shift on 10011001:

(a) 01001100
(b) 11100110
(c) 00011001
(d) 11001100

Q6. Which logic micro-operation is used to selectively clear bits of a register?

(a) OR
(b) XOR
(c) AND
(d) NOT

Q7. An arithmetic right shift is used to:

(a) Multiply by 2
(b) Divide by 2 while preserving sign
(c) Rotate bits
(d) Clear register

Q8. In basic binary multiplication, the multiplicand is added to the product when:

(a) Multiplier bit is 0
(b) Multiplier bit is 1
(c) Carry is generated
(d) Overflow occurs

Q9. Overflow in signed binary addition occurs when:

(a) Carry into MSB ≠ Carry out of MSB
(b) Carry is 1
(c) Result is negative
(d) MSB is 0

Q10. Find A − B using 2’s complement:
A = 00110110, B = 01001101

(a) 11101001
(b) 11001001
(c) 00101001
(d) 10101001

Answers:

| 1.(b) | 2. (b) | 3.(b) | 4.(b) | 5.(d) | 6.(c) | 7.(b) | 8.(b) | 9.(a) | 10.(a) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |