1. In a system using isolated I/O, which of the following statements is INCORRECT?

A. I/O devices have a separate address space
B. Special instructions like IN and OUT are required
C. Memory instructions can directly access I/O devices
D. Separate control lines exist for memory and I/O

Ans: C

2. Which condition is most likely to reduce CPU idle time during slow I/O operations?

A. Increasing CPU clock speed
B. Using programmed I/O
C. Using interrupt or DMA
D. Increasing buffer size only

Answer: C

3. Which of the following is NOT a function of an I/O interface?

A. Instruction decoding
B. Synchronization between CPU and peripherals
C. Data format conversion
D. Signal level matching

Answer: A

4. Why is data conversion required in I/O interface?

A. CPU uses analog signals
B. Peripherals and CPU operate with different data units
C. Memory cannot store binary data
D. Bus width mismatch only

Answer: B

5. A peripheral device transfers data serially, while CPU processes data in parallel words. What is primarily required?

A. Clock synchronization only
B. Data format conversion
C. Bus arbitration
D. Cache memory

Answer: B

6. Which register in I/O interface is responsible for indicating errors and device status?

A. Control register
B. Data register
C. Status register
D. Address register

Answer: C

7. If CPU executes an I/O read command, what happens?

A. CPU sends data to peripheral
B. Interface sends data to peripheral
C. Memory sends data to CPU 
D. CPU receives data from interface buffer

Answer: D

8. Why are special I/O instructions (IN/OUT) considered less flexible?

A. They consume more power
B. They restrict addressing modes and operations

C. They cannot access memory
D. They are slower than MOV

Answer: B

9. In a system, the CPU executes a MOV instruction to communicate with an I/O device. What can be concluded?

A. System uses isolated I/O
B. System uses memory-mapped I/O
C. System uses DMA
D. Cannot be determined

Answer: B

10. Which situation does NOT require an I/O interface?

A. CPU communicating with RAM
B. CPU communicating with keyboard
C. CPU communicating with disk
D. CPU communicating with printer

Answer: A

11. Why is synchronization required even if data formats match?

A. Because of voltage mismatch
B. Because CPU is always slower
C. Because devices operate at different speeds
D. Because bus width differs

Answer: C