Q.1 In a system using software polling priority interrupts, which of the following conditions leads to the maximum latency for servicing a high-priority device?

A. When polling order is dynamically changed
B. When all devices are simultaneously requesting interrupts
C. When the high-priority device is placed last in polling sequence
D. When interrupt vectoring is used

Answer: C

Q.2 In a daisy chain priority interrupt system, if a device has PI = 1 and PO = 1, what does it indicate?

A. Device is requesting interrupt and has highest priority
B. Device is not requesting interrupt and passes priority
C. Device is requesting interrupt but not granted priority
D. Device is blocked by CPU

Answer: B

Q.3 In a hardware priority interrupt system, the priority encoder output (x, y) is determined by inputs I0,I1,I2,I3I_0, I_1, I_2, I_3I0​,I1​,I2​,I3​.
If I2=1I_2 = 1I2​=1 and all higher priority inputs are 0, what will be the encoded output?

A. (0,1)
B. (1,0)
C. (1,1)
D. (0,0)

Answer: B

Q.4 Which of the following is NOT true about mask registers in priority interrupt systems?

A. They selectively disable interrupt requests
B. Each bit corresponds to an interrupt source
C. They determine priority order dynamically
D. They can be modified using instructions

Answer: C

Q.5 During an interrupt cycle, which of the following sequence is correct?

A. PC ← VAD → Push PC → Disable IEN
B. Push PC → PC ← VAD → Disable IEN
C. Disable IEN → Push PC → PC ← VAD
D. Push PC → Disable IEN → PC ← VAD

Answer: B

Q.6 In parallel priority interrupt, what is the role of INTACK signal?

A. Clears interrupt register
B. Enables device to send data
C. Loads vector address into PC
D. Enables priority logic to place vector address on bus

Answer: D

Q.7 Which condition must be satisfied for CPU to enter interrupt cycle?

A. IEN + IST = 1
B. IEN · IST = 1
C. IEN ⊕ IST = 1
D. IEN = IST

Answer: B

Q.8 Which of the following is a major disadvantage of software polling over hardware priority interrupts?

A. Requires complex hardware
B. Cannot handle multiple interrupts
C. Slow due to sequential checking
D. Cannot assign priority

Answer: C

Q.9 In a polling-based priority interrupt system with 6 devices, each requiring 2 µs to check, what is the worst-case delay before servicing the highest priority device if it just missed being checked?

A. 2 µs
B. 10 µs
C. 12 µs
D. 6 µs

Answer: C

Q.10 In the initial sequence of interrupt service routine, why is IEN set to 1 again after saving registers?

A. To terminate interrupt
B. To allow nested interrupts
C. To clear interrupt request
D. To reset priority encoder

Answer: B

Q.11 For the instruction LDA, which sequence correctly represents microoperations?

A. DR ← M[AR], AC ← DR
B. AC ← M[AR], DR ← AC
C. DR ← AC, AC ← DR
D. AC ← AR

Answer: A

Q.12 In BUN (Branch Unconditionally) instruction, the program counter is updated as:

A. PC ← PC + 1
B. PC ← AR
C. PC ← DR
D. PC ← M[AR]

Answer: B

Q.13 Which instruction involves storing return address before branching?

A. BUN
B. ISZ
C. BSA
D. STA

Answer: C

Q.14 During execution of a memory reference instruction, if D7 = 0 and I = 1, which operation must occur at T3T_3T3​?

A. Execute register reference instruction
B. Execute I/O instruction
C. Fetch effective address from memory
D. Increment PC

Answer: C

Q.15 In the instruction cycle, indirect addressing requires an additional memory access at:

A. T1
B. T2
C. T3
D. T4

Answer: C

Q.16 Which instruction results in no change in AC but affects control flow?

A. ADD
B. LDA
C. BUN
D. STA

Answer: C

Q.17 If AC = 1111 1111 1111 1111 (−1) and DR = 1, after ADD operation:

A. AC = 0, E = 1
B. AC = 0, E = 0
C. AC = 1, E = 1
D. AC = −1, E = 0

Answer: A

Q.18 In memory reference instructions, the effective address (EA) is stored finally in:

A. PC
B. IR
C. AR
D. DR

Answer: C

Q.19 Which of the following scenarios leads to two memory accesses before execution phase?

A. Direct addressing ADD
B. Indirect addressing LDA
C. STA instruction
D. BUN direct

Answer: B

Q.20 In a hardware priority interrupt system, if multiple interrupts occur simultaneously and some are masked, the priority resolver will:

A. Ignore all interrupts
B. Select highest priority among all interrupts
C. Select highest priority among unmasked interrupts
D. Select lowest priority interrupt

Answer: C