What is the primary function of the Program Status Word (PSW) register?
A. To store the address of the next instruction.
B. To store the current instruction being executed.
C. To store arithmetic flags and CPU status bits.
D. To store the temporary data during arithmetic operations.

Answer: C

The Program Status Word (PSW) is also known as: **
A. Instruction Register
B. Flag Register
C. Address Register
D. Accumulator

Answer: B

Which of the following is NOT a status flag typically found in a PSW?
A. Zero Flag (ZF)
B. Carry Flag (CF)
C. Parity Flag (PF)
D. Interrupt Pointer (IP)

Answer: D

When a CPU detects an interrupt, what does it typically do with the current PSW?
A. Clears it.
B. Saves it in the accumulator.
C. Saves it on the stack.
D. Overwrites it.

Answer: C

Which bit in the PSW indicates that the result of a signed arithmetic operation is outside the range?
A. Z
B. C
C. V
D. P

Answer: C

If the result of a subtraction in the accumulator is zero, which flag will be set in the PSW?
A. C
B. Z
C. V
D. None of these

Answer: B

A branch instruction that transfers control to a new address regardless of any condition is called:
A. Conditional Branch
B. Unconditional Branch
C. Subroutine Call
D. Return
Answer: B

What is the main purpose of an interrupt in program control?
A. To increase memory size
B. To suspend current execution for a higher-priority task
C. To turn off the computer
D. To speed up calculations
Answer: B

Which mechanism is used to return from a subroutine?
A. Interrupt
B. Jump
C. Return/Pop
D. PUSH
Answer: C

In many processors, the Program Status Word (PSW) is automatically updated after the execution of arithmetic instructions. Which of the following statements is NOT necessarily true?

A. The carry flag is affected by unsigned arithmetic operations.
B. The zero flag becomes set if the result of an operation is zero.
C. The overflow flag indicates an error in both signed and unsigned arithmetic.
D. The sign flag reflects the MSB of the result.

Answer: C

Assume a processor where the PSW contains flags Z (zero), C (carry), S (sign), and V (overflow). After executing the instruction:

(01111111)2+ (00000001)2​

(using 8-bit signed representation), which flags will be set?

A. C and Z
B. V and S
C. C and V
D. Z and V

Answer: B

Which of the following operations does NOT modify the PSW flags in most architectures?

A. ADD
B. SUB
C. MOV
D. CMP

Answer: C

In a typical processor, a conditional branch instruction uses the PSW to determine whether the branch should occur. Which hardware component is primarily responsible for evaluating this condition?

A. Control Unit
B. Instruction Register
C. Program Counter
D. Arithmetic Logic Unit

Answer: A

Suppose an instruction SUB R1, R2 performs R1−R2R1 - R2R1−R2. After execution, the PSW flags indicate Z = 0, S = 1, V = 0, C = 1. Which interpretation is correct?

A. The result is negative with borrow.
B. The result is positive without borrow.
C. The result is negative without overflow.
D. The result is zero but borrow occurred.

Answer: C

A processor executes a conditional jump if zero (JZ) instruction. Which PSW flag must be examined for this instruction to branch?

A. Carry flag
B. Sign flag
C. Zero flag
D. Overflow flag

Answer: C

A conditional branch instruction is executed, but the branch condition is false. What happens to the Program Counter (PC)?

A. PC remains unchanged
B. PC loads the branch target address
C. PC increments to the next sequential instruction
D. PC stores the return address

Answer: C

In a typical subroutine call instruction, which of the following operations occurs first?

A. Load subroutine address into PC
B. Save return address in stack or register
C. Update PSW flags
D. Flush instruction pipeline

Answer: B

Which of the following instructions is generally used to compare two operands and affect branching without storing the result?

A. SUB
B. CMP
C. TEST
D. MOV

Answer: B

In many processors, the return instruction (RET) in a subroutine typically performs which action?

A. Loads PC with value stored in stack
B. Restores PSW flags only
C. Clears stack pointer
D. Loads PC with current instruction address

Answer: A

Which of the following distinguishes an interrupt from a subroutine call?

A. Interrupts are always software initiated
B. Subroutines are asynchronous events
C. Interrupts automatically save processor context
D. Subroutines always modify PSW

Answer: C

Suppose a processor executes a CALL instruction located at memory address 200, and the instruction length is 3 bytes. What value is typically pushed onto the stack?

A. 200
B. 201
C. 203
D. Address of the first instruction in the subroutine

Answer: C

Which of the following types of branching always changes the flow of execution regardless of flag conditions?

A. Conditional branch
B. Relative branch
C. Unconditional branch
D. Indexed branch

Answer: C

An interrupt occurs while a program is executing. Which sequence of actions is generally performed by the processor?

A. Disable interrupts → save PC → jump to interrupt vector
B. Save PC and PSW → load ISR address → execute interrupt service routine
C. Jump to ISR → save registers → disable interrupts
D. Save stack pointer → clear PSW → restart program

Answer: B

Which of the following addressing modes is most commonly used in branch instructions?

A. Immediate addressing
B. Relative addressing
C. Indirect addressing
D. Register addressing

Answer: B

In vectored interrupt systems, how does the processor determine the address of the interrupt service routine (ISR)?

A. The ISR address is stored in PSW
B. The interrupting device supplies a vector number used to locate the ISR
C. The ISR address is stored in PC automatically
D. The ISR address is fixed for all interrupts

Answer: B